## Assembly Register Basics

## 1. Introduction

Registers are small, high-speed storage locations inside the CPU. They are used to store  data temporarily while a program is running. Registers are much faster than RAM and are directly accessed by the processor during instruction execution.

# 2. Basic Working Model (Input → Process → Output)

### 🔹 Input

Data is loaded into registers from:

- Keyboard

- Memory (RAM)

- File

- Immediate values (numbers written in code)

Example:

MOV AX, 5

Here, 5 is input into register AX.

### 🔹 Process

The CPU performs operations using registers:

- Addition

- Subtraction

- Multiplication

- Division

- Logical operations

Example:

ADD AX, BX

The processor adds values stored in AX and BX.

### Output

The result stored in registers is:

- Sent to memory

- Displayed on screen

- Returned to operating system

Example:

mov eax, 1

int 0x80

# 3. Types of Registers (8086 / x86 Overview)

## 3.1 General Purpose Registers

Used for arithmetic and data storage.

| Register | Description |
| --- | --- |
| AX (Accumulator) | Used in arithmetic operations |
| BX (Base) | Used as base pointer for memory access |
| CX (Counter) | Used in loops and counting |
| DX (Data) | Used in multiplication/division and I/O |

### 16-bit Sub Registers

| Register | Hight Byte | Low Byte |
| --- | --- | --- |
| AX | AH | AL |
| BX | BH | BL |
| CX | CH | CL |
| DX | DH | DL |

## 3.2 Segment Registers (8086)

Used to access memory segments.

| Register | Purpose |
| --- | --- |
| CS | Code Segment |
| DS | Data Segment |
| SS | Stack Segment |
| ES | Extra Segment |

## 3.3 Pointer Registers

| Register | Purpose |
| --- | --- |
| SP | Stack Pointer |
| BP | Base Pointer |
| SI | Source Index |
| DI | Destination Index |

## 3.4 Instruction Pointer

| Register | Purpose |
| --- | --- |
| IP | Holds Address of next Instruction |

## 3.5 Flags Register

Contains status flags that indicate results of operations:

- Zero Flag (ZF)

- Carry Flag (CF)

- Sign Flag (SF)

- Overflow Flag (OF)